HDMIIPECO firmware upgrade manual

TRANSMITTER

- Connect Transmitter unit to PC with an UTP cable and connect power

- On PC networking settings change PC IP to the same network with Transmitter 192.168.1.xx (xx can be any two numbers except 11 and 12. 11 is reserved for transmitter and 12 is reserved for receiver

- Open internet explorer and type in 192.168.11

- User name: admin  Password: admin

- Choose encoder update first and once completed select update firmware to update firmware

- Once successfully updated encoder firmware and system firmware disconnect unit from PC.

Please DO NOT interrupt updating process!

RECEIVER:

- Connect Receiver to PC via UTP cable exactly the same way as Transmitter and plug in power

- Open internet explorer and type in IP 192.168.1.12

- Choose receiver firmware and upgrade

- An upgrade complete message will show

THIS IS THE INITIAL UPDATE TO ENABLE PERFORMING FUTURE UPDATES THROUGH PC TOOL SOFTWARE.